Standard Operating Procedure

SOP Ref No. CTR/Niakan/010

Version:

SOP title: Inmunosurgery

Project supervisor Kathy Niakan

Author Claire Simon 23/08/2022

Please acknowledge our work if you use or adapt these protocols in your research.

Purpose

The objective of this Standard Operating Procedure is to describe the immunosurgery technique used to selectively lyse trophectoderm cells from blastocysts using antibody and complement treatment, enabling isolation of the inner cell mass (ICM).

Materials and equipment

| PPE | Consumables | Equipment |
| --- | --- | --- |
| - Gloves - Lab coat - Goggles/face shield - Closed footwear | - Guinea Pig Serum Complement (Merk, Cat. 234395) - Rabbit anti-mouse serum (Sigma, Cat. M5774) - Rabbit anti-human serum (Sigma, Cat. H8765) - Global medium + 10% HAS (Origio, H5GT-030) - Acid Tyrode’s solution (Sigma, Cat. T1788) - Mineral oil (Origio, Cat. ART-4008-5P). - 6 cm tissue culture plates - Pulled glass capillaries - 75 µm Stripper tips (Origio). | - 37°C incubator with 5% CO₂ - Microscope - Heating block or water bath - Ice bucket |

Procedure

Preparation of the reagents

| Name | Company | Cat no. | Storage |
| --- | --- | --- | --- |
| Guinea Pig Serum Complement | Merk | 234395 - 5ML | Reconstitute 5ml ice cold distilled water – 100µl & 200µl aliquots -80°C (sensitive to temp, thaw immediately before use) |
| Rabbit anti-mouse serum | Sigma | M5774 - 2ML | 100µl aliquots -80°C |
| Rabbit anti-human serum | Sigma | H8765 - 2ML | 100µl aliquots -80°C |

Notes

It is essential that the time between thawing complement and its use is kept to a minimum because it is highly unstable.

Immunosurgery is dependent on having an intact trophectoderm epithelium, to provide a barrier to the antibodies/complement. Handle embryos gently, and do not expose to extreme changes in temperature (i.e. pre-incubate all reagents). If the embryos collapse, the trophectoderm layer is likely to be leaky. If this happens, incubate embryos in regular medium to allow the trophectoderm epithelium to reform, and the blastocyst cavity should re-expand. Immunosurgery should not be performed on embryos with visibly damaged trophectoderm.

Immunosurgery is inefficient, and its success is dependent on the operator. Do not expect complete removal of TE, but rather a significant depletion. https://doi.org/10.1016/j.diff.2011.04.001

Preparation

Before starting equilibrate medium, mineral oil, and acid Tyrode’s solution in 37°C incubator 5% CO2 (minimum 30min)

Thaw anti-mouse serum or anti-human serum at RT, thaw complement on ice.

Procedure (Modified from Strawbridge, Clarke, Guo & Nichols 2021)

- Prepare 20% anti-mouse or anti-human serum. Incubate plate to equilibrate (min 30min) before use.

400µl Global medium + 10% HSA

100µl anti-mouse serum or anti-human serum

Medium

6cm plate

15µl drops

Under oil

Incubate before use

Under oil

Incubate before use

20% anti-mouse/human serum

- Remove zona pellucida by washing embryos through 3 pre-warmed drops of AT (~50µl drops)

- Wash in 3x drops Global medium + 10% HSA.

If any embryos have collapsed, incubate for ~20 min to allow re-expansion

- Wash embryos through 3 x drops of 20% anti-mouse/human serum

- Move embryos to 4th drop of 20% anti-mouse or anti-human serum and incubate for 30 min at 37°C

- Wash through 4x drops of Global medium

If any embryos have collapsed, incubate for ~20 min to allow re-expansion

- During incubation prepare plate for immediate use with 20% Guinea pig serum complement, incubate until use.

400µl Global medium + 10% HSA

100µl Guinea pig complement

6cm plate

15µl drops

Under oil

Incubate before use

Medium

20% complement

- Wash embryos through 3x drops of 20% complement

- Move embryos to 4th drop of 20% complement and incubate at 37°C for 10-20min

Timing is very sensitive! Too little time and you won’t get lysis, inefficient immunosurgery, and significant trophectoderm carry-over. Too much time and the entire embryo will lyse. Monitor the embryos to ensure TE lysis. Outer trophectoderm cells should be swollen as they lyse, this will make the embryo appear bubbly on the surface. For mouse embryos this occurs within ~10 min. For human embryos, it takes longer, up to ~20min. Check under the microscope, continue incubation if do not observe lysis. See Strawbridge et al. 2021 for example images.

- Wash embryos through 3x drops of medium

- Move embryos into the 4th drop of medium and incubate for 10min to allow for the trophectoderm to fully lyse/die.

- Triturate with pulled capillary to remove lysed TE.

I remove lysed TE cells with 75um Stripper tip and/or pulled glass capillary. For initial steps find a Stripper tip or capillary diameter that is smaller than the embryo, but larger than the ICM. Initial trituration should aim to break off large chunks of TE (mainly mural), with some polar TE remaining. Lysed TE cells will also slough off during trituration. If polar TE cells remain, using a pulled glass capillary with a diameter of ~1-2cells, and much smaller than the ICM, use this to carefully remove TE cells from the surface of the ICM. Do not draw the ICM into this narrow bore, as you  risk damage / breaking up of the ICM.